HOW TO USE

Right click ‘menu.fig’ and select ‘Open in GUIDE’.

You will see the GUI template of the algorithm, but it is not functional.

Either click ‘Run Figure’ on the upper menu or use key combination: Ctrl + T.

The GUI of the algorithm (the usable one) will appear.

THE GUI OF THE ALGORITHM:

NOTE: The textboxes, radiobuttons and checkboxes could be changed during run-time, but this is not advised since this may cause a failure in the run.

1) # of Components Range:

(has effect only if parametric_solution_filename is checked)

The algorithm tries to kill as many components as possible. (

to kill = to disable, to make gain value 0

components = gain components: g1,g2,… etc.

to save = opposite of  ‘to kill’, not forcing a component to be 0

WARNING: The value of a component may be 0 in a solution (parametric or numeric) even though it was not forced to be 0.

“# of Components Range” area is about determining the # of alive components the algorithm will run for. Assuming there are N components:

1-a) ALL # of Components: The algorithm will start by saving 1 component (killing N-1) and trying to find all possible parametric solutions, then it will save 2 (kill N-2) and try to find all possible parametric solutions,… , then it will save N (killing none) and try to find all possible parametric solutions. The algorithm ends when # of saved components reach N.

This choice may consume A LOT OF time!

1-b) N_COMPONENTS_MORE from Start: In this mode, the algorithm will start by saving “Starting # of Components” components (killing the rest), try to find all possible parametric solutions, then will save “Starting # of Components+1” components(killing the rest), try to find all possible solutions,… This goes on.

The algorithm will end EITHER (when the # of saved components reach N) OR (when the # of saved components reach L+ N_COMPONENTS_MORE).

L: Least # of components needed to be saved in order to find a solution

For example, the desired STF and NTF’s may be impossible to realize with very few components, but may be possible with more components.

For example, for 2nd order generic sigma-delta ADC converter netlist, for OUT_STF =(1/z)2 and OUT_NTF=(1-z-1)2, there is no solution for 4 components, but there are 2 parametric solutions for 5 components. In this case, the algorithm will run for 5,6,7,…,(5+ N_COMPONENTS_MORE) components.

This is a good automated mode which does not only finds solutions, but also finds more solutions for different number of components, which diversify the solutions so we will have different solutions to choose the one that is best for our purpose. This mode also usually needs MUCH less time than “ALL # of Components” mode.

This mode may be used to run the algorithm for a specific # of components, say M. In order to do this, choose “Starting # of Components”= M and N_COMPONENTS_MORE=0.

1-c) N_COMPONENTS_MORE in Range: This mode is quite similar to “N_COMPONENTS_MORE from Start” mode but this time the algorithm will end when the # of saved components reach “Ending # of Components”.

So, the algorithm will end EITHER (when the # of saved components reach N) OR (when the # of saved components reach L+ N_COMPONENTS_MORE) OR (when the # of saved components reach “Ending # of Components”).

L: Least # of components needed to be saved in order to find a solution

This mode may be used to run the algorithm for a specific # of components, say M. In order to do this, choose “Starting # of Components”= “Ending # of Components” = M and N_COMPONENTS_MORE=0.

2) Component Kill Combination Range:

(has effect only if parametric_solution_filename is checked)

This area determines for which component kill combination range the algorithm will run for.

In order to be sure to find all the solutions, “ALL Combos” must be selected.

But in some cases, if we need to compare some results or find solutions for specific cases, COMBOS in RANGE may be selected and START_COMBO and END_COMBO may be specified.

It is easier to explain this on an example:

I want to find the solution (if it exists) for g2=g5=g6=g9=g10=g13=0, for standard 2nd order sigma-delta ADC circuit, which has 15 components. This means 6 components killed and 9 components are saved.

With some trial&error search using MATLAB’s nchoosek function, I find out that this combination is 2879th combination of all 5005 combinations (see below for method to determine that number).

I choose “N_COMPONENTS_MORE in Range” mode, speficy

“Starting # of Components”=“Ending # of Components”=9, N_COMPONENTS_MORE=0.

I choose “Component Kill Combination Range” mode instead “ALL Combos” mode and specify START_COMBO=2879 and END_COMBO=2879.

When I click “START”, the algorithm runs just for 9 components alive at 2879th combination, which is exactly the condition I wanted.

I get the solution, which is:

g1=z11/(2*z)

g2=0

g3=1/2

g4=z

g5=0

g6=0

g7=-4

g8=(8*z)/z11

g9=0

g10=0

g11=-1/2

g12=z11

g13=0

g14=1/(4*z)

g15=(4*z)/z11

3)netlist_filename:

Just the name name of the netlist file. For example: netlist_file.txt

4)condition_filename:

Just the name name of the condition file. For example: condition _file.txt

5) trial_list  & trial_list_filename:

Trial list can either be entered directly into the textbox or trial list filename may be specified. In either case, the trial list format is the same. Just numbers separated by spaces, and do NOT put an extra space at the end of line.

6) Save TFs: Filename:

If the checkbox is checked, the algorithm saves TFs of all nodes to a file whose name is specified in the textbox.

NOTE: The output format is:

(node_STF)*IN+(node_NTF)*Er

WARNING: For giving easy-to-understand outputs, collect and simplify functions of MATLAB are called in this procedure. These functions may need some time, and saving TFs has only one use: Helping the user understand the circuit better. On the other hand, this program may be used just to analyze a circuit and gather TFs.

7) Solve Parametric: Filename:

If the checkbox is checked, the algorithm will find parametric solutions and write it into the file named in the textbox.

The algorithm steps are:

1) Read the netlist & create node equations from netlist

2) Solve node equations & find node TF’s

3) Read desired STF & NTF

4) Match desired STF & NTF with node TF’s and create component equations to solve

5) Solve component equations to find parametric solutions

6) Write parametric solutions into file whose name was specified in the textbox

Node equations are in terms of inputs (IN,Er) and nodes (OUT,x1,x2,…).

Example:

x1=x2+x2

Component equations are in terms of components (g1,g2,…)

Example:

(g1*g2)/(g4+1)=1

8) Solve Numeric: Filename:

If the checkbox is checked, the algorithm will find numeric solutions and write it into the file named in the textbox.

The algorithm steps are:

1) Acquire trial list from EITHER from textbox OR from file whose name was specified

2) Read the parametric solution file

3) Find all trial list combinations of numeric solutions

4) According to criteria_1, create solution pool of size SOLUTION_POOL_SIZE

5) According to criteria_2, find the best solution in the solution pool

6) Add the best solution to the numeric solution array

7) Get rid of duplicate numeric solutions

8) Write numeric solutions into file whose name was specified in the textbox

NOTE: criteria_1 & criteria_2 are two criteria for numeric solutions to find the better ones.  For now, “criteria_1” is absolute value of (maximum gain)/(minimum gain) in a numeric solution. “criteria_2” is the sum of absolute values of all gains in a numeric solution. If you want first to create solution pool according to criteria_2 and then get the best according to criteria_1, just swap the function names and names of .m files. You can also modify or rewrite criteria_1 & criteria_2 completely.

WARNING: The aim is to minimize both criteria_1 & criteria_2, so the solution pool and the best solution is selected by MINIMIZING criteria_1 & criteria_2! If you plan to change criteria_1 & criteria_2, you have to keep this in mind.

NOTE: “best solution” = Best numeric solution found for that parametric solution. So, there are more than 1 “best solution”s. In fact before clearing duplicates, there are exactly the same number of “best solution”s as parametric solutions. (Since there is one for each)

9) SOLUTION_POOL_SIZE:

SOLUTION_POOL_SIZE is used to create a solution pool which has size of “SOLUTION_POOL_SIZE” solutions. Changing this number may sometimes change the best solutions found at the end of Solve Numeric operation.

10) START & STATUS:

Solve Parametric & Solve Numeric are completely separated operations, so you can run only Solve Parametric and find & store parametric solutions into a file OR you can run only Solve Numeric to find & store numeric solutions into a file OR run both Solve Parametric and Solve Numeric to do both of them respectively.

START button simply starts algorithm to run. The algorithm runs as specified in other radioboxes, checkboxes and textboxes in the GUI.

STATUS gives information about the current operation.

There are 11 different STATUS messages:

1  ) IDLE

This message can only be seen when the GUI was first opened. The algorithm could be run as many times as desired without closing the GUI, but this message never appears again.

2  ) SOLVING NETLIST

Includes steps 1,2 of Solve Parametric.

3  ) SAVING TFs TO FILE

This message is shown when the TFs of all nodes are being saved into a file.

4  ) MATCHING EQUATIONS

Includes steps 3,4 of Solve Parametric.

5  ) FINDING PARAMETRIC SOLUTIONS

Includes step 5 of Solve Parametric.

6  ) PRINTING PARAMETRIC SOLUTIONS TO FILE

Includes step 6 of Solve Parametric.

7  ) Cannot find desired responses with this topology!

This message is shown when some of the desired STF or NTF have higher order terms of ‘z’ (of z-domain, not parameter) than node STF and NTF. In this case it is impossible to find solutions since we will have an equation like “K=0” in the component equations, where K is not 0. So that equation system has no solutions, regardless of the combinations tried. In fact, killing components can only reduce node STF or NTF orders, so it is impossible to match desired STF or NTF which is a higher order polynomial.

8  ) FINDING NUMERIC SOLUTIONS

Includes steps 1,2,3,4,5,6 of Solve Numeric.

9  ) CLEARING DUPLICATE NUMERIC SOLUTIONS

Includes step 7 of Solve Numeric.

10 ) PRINTING NUMERIC SOLUTIONS TO FILE

Includes step 8 of Solve Numeric.

11) FINISHED

This message is shown after the algorithm is finished running. The computer is idle now.

Apart from STATUS, there may also be some outputs on the terminal giving information about current status.

APPENDIX: FILE FORMATS:

NOTES:

- Format format:

- ### indicates a number.

- The parts written in italic are variables.

- The parts written in bold are constant strings.

- If you use Windows’ notepad to view text files, you may see only one line since notepad does not recognize ‘\n’ character correctly. But if you open the text files in MATLAB’s editor, you will view them correctly.

INPUT FILE FORMATS:

x### corresponds node ###. This convention is valid for whole algorithm.

For all input files, for the sake of safety, do not put any spaces at the end of lines and do not put any extra '\n's at the end of files.

Extra Components

Nodes

Components

Equations

Netlist Text File Format:

This text file determines the circuit by its netlist.

Format:

Each line format depends on component type.

GAIN component:

Format:

GAIN### NODE1 NODE2

Where ### is a number indicating the indice of the component.

Example:

GAIN1 1 2

adds a GAIN component with indice 1 between nodes 1 and 2. So, the component will be mentioned as g1 in the algorithm, and node 1 and 2 will be mentioned as x1 and x2 respectively.

ADDER component:

Format:

ADDER SIGNED_NODE_1 SIGNED_NODE_2 ... SIGNED_NODE_N NODE_SUM

adds an ADDER component that adds or subtracts nodes SIGNED_NODE_1 to SIGNED_NODE_N depending on their signs and makes NODE_SUM this sum.

Example:

ADDER 2 -5 -4 -23 3

adds an adder component which does the operation that corresponds to:

x2-x5-x4-x23=x3

INTEGRATOR component:

...

Condition Text File Format:

This text file determines the STF and NTF conditions on nodes. The desired STF and NTF on output & nodes should be entered into this file.

Format:

NODE MODE TF

Where NODE is the node name (x1,x2,etc or OUT)

MODE is either STF or NTF

TF is the desired transfer function on that node.

TF is in z-domain .

Example:

OUT STF 1/(z^2)

adds a condition that STF of OUT node should be 1/(z^2).

Example:

x5 NTF 1/(z^2)

adds a condition that NTF of x5 node should be 1/(z^2).

Trial List Text File Format:

This file contains the numbers to be substituted into parametric solutions in order to find numeric solutions. The algorithm tries all combinations of trial list members when finding numeric solutions.

NOTE: Having a longer trial list will increase the time needed to calculate numeric solutions very much! This is because the # of numeric solutions to be found per parametric solution is:

# of numeric solutions=(# of elements in trial list)^ (# of parameters in that parametric array)

So having 2 parameters with 5 elements in trial list means there will be 25 numeric solutions (to find the best among) for that parametric solution. But in some cases there are more parameters. If there are 4 parameters, with 5 elements in trial list there will be 625 numeric solutions. But if there are 9 elements in the trial list (for example: trial_list=[1/5 1/4 1/3 1/2 1 2 3 4 5]) there will be 6561 numeric solutions.

Format:

trial_list_member_1 trial_list_member_2 ... trial_list_member_N

This file just contains some numbers seperated by space.

Example:

1/3 1/2 1 2 3

OUTPUT FILE FORMATS:

TF File Format:

NODE_NAME = ( NODE_STF )*IN+(NODE_NTF)*Er

Parametric Solution File Format:

COMPONENT_1 COMPONENT_2 ... COMPONENT_N

SOLN_1_COMPONENT_1 SOLN_1_COMPONENT_2 ... SOLN_1_COMPONENT_N

SOLN_2_COMPONENT_1 SOLN_2_COMPONENT_2 ... SOLN_2_COMPONENT_N

.                  .                      .

.                  .                      .

.                  .                      .

SOLN_M_COMPONENT_1 SOLN_M_COMPONENT_2 ... SOLN_M_COMPONENT_N

NOTE : The solutions may (and usually) be parameterized by “z” and “z###”s (z with some indices, for example: z15 or z112). This “z” here has no relation with the z-domain. This “z” and “z###”s are just parameters. You can do text-replacement: “z”“a”, “z15”“a15”,… (or some other parameter letter that you are comfortable with) in your mind when interpreting the parametric solutions.

Numeric Solution File Format:

COMPONENT_1 COMPONENT_2 ... COMPONENT_N

SOLN_1_COMPONENT_1 SOLN_1_COMPONENT_2 ... SOLN_1_COMPONENT_N

SOLN_2_COMPONENT_1 SOLN_2_COMPONENT_2 ... SOLN_2_COMPONENT_N

.                  .                      .

.                  .                      .

.                  .                      .

SOLN_M_COMPONENT_1 SOLN_M_COMPONENT_2 ... SOLN_M_COMPONENT_N

A CONVENTION NOTE FOR THE CODE :

Plural name means the items are listed in a string separated by commas.

Ex.: nodes has value something like this: ‘x1,x2,x3,x4’

Name ending with “_array” means it is an array. It is usually a cell array. Please note that there is not any string arrays in my code.

Ex.: node_array has value like this: [{‘x1’} {‘x2’} {‘x3’} {‘x4’}]

NOTE: {} indicates a cell in MATLAB.

Plural name ending with “_array” means that variable is an array bur array elements also has multiple elements in them listed as string, separated by commas.

Ex.: equations_array has value like this: [{‘g1=g2,g3=g4+g5’} {‘g7=g8+g9’} {‘g1=g6,0=0,2=2’}]